KEYAN Calculation — Technical Support Documentation

CAE Simulation Troubleshooting & Error Knowledge Base

1. Purpose & Troubleshooting Framework

Advanced FEA and multiphysics solvers (e.g., ANSYS Workbench, mechanical solvers) frequently encounter numerical divergence due to structural non-linearities, material discontinuities, or improper constraint setups. This Knowledge Base serves as a self-service diagnostic index for engineers to rapidly decode solver error logs, isolate root causes, and execute standardized corrective workflows.

2. Core Solver Error Matrix & Resolution Index

The following matrix catalogs the most critical errors encountered during high-temperature thermal-structural and non-linear simulation runs, paired with actionable engineering countermeasures:

| Error Code / Type | Typical Log Message | Root Cause Analysis | Mandatory Solution Workflow |
| --- | --- | --- | --- |
| Solver Divergence (Non-linear) | Solution magnitude/increment has exceeded the limit. Solution not converged. | Extreme material non-linearity at weld pools (2200°C) or structural buckling under high thermal strain. | 1. Activate Auto Time Stepping. 2. Shrink initial time-step to 0.001s. 3. Turn ON Large Deflection effects. 4. Apply non-linear stabilization damping (Factor = 1e-4). |
| Rigid Body Motion (Under-constrained) | Solver pivot error. One or more joints/nodes may have unconstrained rigid body motion. | An assembly part lacks sufficient mechanical boundary conditions or active contacts, causing infinite displacement. | 1. Run an initial Modal Analysis to find 0-Hz unconstrained modes. 2. Ensure Shared Topology or bonded contacts connect all sheet metal plates. 3. Add weak springs for stabilization. |
| Contact Penetration (Contact Chatter) | Excessive penetration detected between contact pairs. Solver aborted. | Contact stiffness is too high or initial clearance causes elements to interpenetrate dramatically in a single load step. | 1. Change Contact Formulation to 'Pure Penalty' or 'Augmented Lagrange'. 2. Set Contact Stiffness to a lower factor (0.01 - 0.1). 3. Enable 'Adjust to Touch' initialization. |
| Distorted Elements (Negative Volume) | Element shape checking failed. Severe mesh distortion detected. | Large structural deformations or high thermal gradients warp first-order or low-quality mesh cells into invalid shapes. | 1. Re-mesh the area using 2nd-Order Hexahedral cells. 2. Refine local element size at high-gradient zones. 3. Check geometric shared topology for redundant edges. |

3. HPC & Hardware Allocation Troubleshooting

Large-scale multi-scale simulation runs require High-Performance Computing (HPC) parallel cluster processing. Hardware-level runtime crashes can be mitigated with the following setups:

Out-of-Memory (OOM) Errors: If the solver aborts with an 'insufficient memory allocations' log, switch the Solver Memory Allocation option from 'In-Core' (keeps everything in RAM) to 'Out-of-Core' (utilizes high-speed NVMe scratch disks for matrix factorization arrays).

MPI Parallel Communication Failures: When utilizing multi-node cluster parallelization, if inter-node communication drops, force the environment to utilize Intel MPI or MS MPI by appending the startup execution flag: '-mpi intel' or '-mpi msmpi'. Disable local firewalls on port ranges 2100-2200.

GPU Acceleration Crashes: Ensure shared acceleration hardware uses verified Enterprise GPU compute drivers (CUDA computing mode). For non-linear mechanical sparse solvers, utilize the startup execution flag '-acc nvidia' only if memory allocations fit the on-card VRAM.

CRITICAL INFRASTRUCTURE SUPPORT: If error logs indicate persistent system-level segmentation faults or licensing server disconnects (-15, -97 errors), please export your complete solver text output log (.wout / .log) and route it to clustersupport@keyan-calculation.com for priority hardware allocation assistance.